# OT Hardware & Software Vulnerability Assessment — Q&A Reference

A working reference for hardware and software vulnerability assessment of

operational-technology (OT) devices: PLCs, RTUs, IEDs, protection relays,

gateways, and embedded field controllers. Written for authorized assessment

work — lab evaluation, red-team engagements with scope, procurement testing,

and academic research. It assumes you have legal authority and physical

possession of the device under test (DUT).

> **Authorization first.** Every technique below assumes a signed scope or an

> owned device. Probing, desoldering, or dumping firmware from equipment you do

> not own or are not authorized to test is a crime in most jurisdictions and can

> also destroy safety-critical evidence. When in doubt, stop and get it in

> writing.

---

## 1. Scope & Methodology

### Q: How does OT hardware/software assessment differ from IT pentesting?

**A:** The priorities invert. In IT, confidentiality usually leads; in OT the

order is **safety → availability → integrity → confidentiality**. Consequences

are physical (pressure loss, overpressure, equipment damage, injury), so the

first rule is *do no harm to the process*. Practical differences:

- **No scanning live plants casually.** A stray Nmap `-sV` or Modbus write can

crash a legacy PLC or trip a process. Fingerprinting is done on bench units,

not production.

- **Devices are long-lived.** 15–25 year service life means unpatched CPUs,

deprecated crypto, and vendors who no longer exist.

- **Vendor cooperation is rare.** You usually work without source, without a

datasheet for the custom silicon, and without an SDK.

- **Physical access is often the whole game.** Field cabinets, exposed serial

ports, and unauthenticated engineering protocols mean the hardware attack

surface matters more than in most IT work.

### Q: What are the standard phases of a device assessment?

**A:** A common flow:

1. **Reconnaissance / OSINT** — model numbers, FCC ID, firmware versions,

vendor advisories, known CVEs, existing teardowns.

2. **External inspection** — ports, interfaces, labels, enclosure, tamper

evidence.

3. **PCB teardown & component ID** — read the board, identify ICs, find debug

and memory interfaces.

4. **Interface identification** — locate UART, JTAG/SWD, SPI, I²C, boot config.

5. **Firmware acquisition** — via update packages, flash dump, or debug port.

6. **Firmware analysis** — unpack, disassemble, hunt for secrets, weak crypto,

backdoors, insecure update logic.

7. **Runtime / protocol testing** — engineering protocols, auth, fuzzing on a

bench unit only.

8. **Reporting** — with reproducible steps, CVSS/CWE mapping, and OT-aware

risk framing (physical consequence, not just CIA).

### Q: What frameworks and standards should anchor the work?

**A:**

- **ISA/IEC 62443** — the OT security backbone; 4-1 (secure development

lifecycle) and 4-2 (component requirements) are the ones you map findings to.

- **MITRE ATT&CK for ICS** — technique taxonomy for the OT-specific behaviors.

- **NIST SP 800-82** — Guide to OT Security (process, risk framing).

- **NIST SP 800-115** — technical assessment methodology (general).

- **OWASP Firmware Security Testing Methodology (FSTM)** and **OWASP IoT Top 10**

— practical firmware/embedded checklists.

- **CWE / CVSS** — for classifying and scoring findings. Consider environmental

CVSS to reflect OT impact.

---

## 2. Reconnaissance & Firmware (Software Side)

### Q: How do I get the firmware if the vendor won't give it to me?

**A:** In rough order of increasing invasiveness:

1. **Vendor update packages** — download the official firmware/patch bundle.

Often the easiest source; unpack with `binwalk`.

2. **Support portals / field techs** — update files are frequently mirrored,

emailed, or left on FTP.

3. **Debug port dump** — pull it over UART/JTAG (see hardware section).

4. **In-circuit flash read** — SPI flash via a clip (e.g., SOIC-8 test clip) and

a `flashrom`-compatible programmer while the CPU is held in reset.

5. **Chip-off** — desolder the flash and read in a socket/programmer. Last

resort; destructive and requires rework skill.

### Q: What's the first thing to do with a firmware image?

**A:**

- `binwalk -E firmware.bin` — entropy plot. Flat high entropy (~7.99) across

the whole image suggests encryption or compression; structured regions

suggest an unencrypted filesystem.

- `binwalk firmware.bin` — signature scan for headers (uImage, SquashFS, JFFS2,

CramFS, gzip, LZMA).

- `binwalk -e` / `unblob` — carve and extract filesystems.

- Identify architecture (ARM/MIPS/PPC/x86) for later disassembly — `file` on

extracted binaries, or CPU markings from the teardown.

### Q: What software vulnerabilities recur in OT firmware?

**A:**

- **Hardcoded credentials / keys / certificates** — search extracted rootfs:

`grep -ri "password\|passwd\|secret\|BEGIN RSA\|BEGIN EC" .`, examine

`/etc/shadow`, `/etc/passwd`, hardcoded WiFi/VPN keys.

- **Insecure update mechanism** — unsigned or unverified firmware, downgrade

attacks, update over cleartext HTTP/TFTP.

- **Weak / deprecated crypto** — DES, MD5, RC4, static IVs, keys shared across

the whole product line.

- **Backdoor / maintenance accounts** — undocumented services, magic packets,

telnet on a hidden port.

- **Vulnerable third-party components** — outdated BusyBox, OpenSSL, web server,

or protocol stacks (map versions to CVEs; build an SBOM).

- **Command injection / buffer overflows** in the web UI and protocol parsers.

- **Secrets in the bootloader environment** (U-Boot `printenv`).

### Q: How do I build an SBOM for a device with no documentation?

**A:** Reconstruct it. Extract the filesystem, enumerate binaries and libraries

with their versions (strings, package manifests, `ldd`-style analysis), and run

the results through a composition-analysis tool (e.g., an SBOM generator that

consumes firmware images). Cross-reference component versions against the NVD.

The output is both a findings source and a deliverable the asset owner can reuse.

---

## 3. Reading Datasheets & Finding Debug/Memory Pins

### Q: I have a PCB and a mystery chip. How do I identify it and get its datasheet?

**A:**

1. **Read the top marking.** Note the manufacturer logo, part number, and

date/lot code. Markings are often abbreviated (top-mark != full ordering

part number) — a marking like `25Q64` maps to a family (Winbond W25Q64,

a 64-Mbit SPI NOR flash).

2. **Decode the logo** if unfamiliar (there are logo-ID references online).

3. **Search the marking** plus package type. For ambiguous top marks, vendor

"top marking" or "marking code" tables in the datasheet disambiguate.

4. **Identify the package** (SOIC-8, WSON-8, TSOP-48, BGA, QFN, QFP) — it

constrains what the chip can be and how you'll interface with it.

5. **Pull the datasheet** and go straight to the **pin configuration /

pinout** and **package/mechanical** sections.

### Q: How do I read a datasheet to locate specific pins?

**A:** The sections you care about, in order:

- **"Pin Configuration and Functions" / "Pinout"** — the diagram showing pin

numbers to signal names. This is your map.

- **"Pin Descriptions" table** — what each named pin does, direction, and

active level (a bar over a name, or a leading `#`/`n`, means active-low, e.g.

`RESET#`, `nWP`, `CS#`).

- **Package drawing** — tells you the physical **pin-1 reference** and pin

numbering order so you can map the datasheet diagram onto the real part.

**Finding pin 1 on the physical package:** look for the **dot/divot** (a molded

circle near pin 1), a **beveled edge or notch**, or a **printed dot** on the

silkscreen. Numbering then runs **counter-clockwise** from pin 1 when viewed

from the top (component side up) for most through-hole and gull-wing SMD parts.

For a standard SOIC-8 with the notch/dot up-left: pin 1 top-left, down the left

side (1→4), then bottom-right up to pin 8.

### Q: Specifically — how do I find the SPI flash pins?

**A:** Most serial NOR flash is an **8-pin SOIC/WSON** using the standard SPI

pinout. On a 25-series part (W25Qxx, MX25Lxx, etc.), the classic pin map is:

| Pin | Name | Function |

|-----|------|----------|

| 1 | CS# / SS | Chip Select (active low) |

| 2 | DO / SO / IO1 | Data out (MISO) |

| 3 | WP# / IO2 | Write Protect (active low) |

| 4 | GND | Ground |

| 5 | DI / SI / IO0 | Data in (MOSI) |

| 6 | CLK / SCK | Serial clock |

| 7 | HOLD# / RESET# / IO3 | Hold/Reset (active low) |

| 8 | VCC | Supply (usually 3.3 V) |

Practical identification workflow:

1. Find the 8-pin chip near the main SoC/CPU — flash sits close to what boots

from it. Read its marking, confirm it's a 25-series SPI NOR (vs. 24-series

I²C EEPROM, which is a different pinout).

2. Confirm against the datasheet pinout for that exact part.

3. The four signals you tap to dump it: **CLK, DI (MOSI), DO (MISO), CS#**,

plus **GND** reference (and VCC if powering externally).

4. To read in-circuit, a **SOIC-8 test clip** to a `flashrom`-supported

programmer (CH341A, a Bus Pirate, an FT2232-based adapter). Hold the host CPU

in reset (or cut its power) so it doesn't fight the bus.

### Q: How do I find JTAG or SWD pins when they aren't labeled?

**A:** JTAG's mandatory signals are **TCK, TMS, TDI, TDO**, plus optional

**TRST** and a reference **GND**/VREF; ARM **SWD** collapses this to **SWCLK**

and **SWDIO** (2-wire) plus ground.

Identification approach:

1. **Look for an unpopulated header or a row/grid of vias/test pads** near the

CPU — often a 0.1"/2.54 mm or 0.05"/1.27 mm footprint (10-pin ARM Cortex

debug, 20-pin classic JTAG, or a bare pad cluster).

2. **Datasheet / reference manual of the CPU** — the SoC datasheet names which

physical pins carry TCK/TMS/TDI/TDO or SWCLK/SWDIO. Trace those to accessible

pads.

3. **Continuity to ground** — a multimeter finds GND pins (0 Ω to chassis/known

ground); that anchors your reference.

4. **Automated pinout discovery** — a **JTAGulator** (or `JTAGenum` on an

Arduino/Pi) brute-forces which pads map to which JTAG signals by walking the

TAP state machine and reading the IDCODE. This is the standard tool when

pads are unlabeled.

5. **Voltage sanity** — most OT targets are 3.3 V logic; some are 1.8 V.

Confirm with a scope/meter before connecting an adapter, or you'll damage

the target or the probe.

### Q: And UART — the fastest first win?

**A:** UART is frequently the easiest foothold (boot logs, a root shell, U-Boot

prompt). It's typically **4 accessible pads**: **TX, RX, GND, VCC**.

- **GND** — continuity to ground.

- **VCC** — steady ~3.3 V (don't drive it; it's a reference/supply, usually

not connected on your adapter).

- **TX** — idles high at the supply rail, then shows **activity bursts at

power-on** (boot log) on a scope/logic analyzer.

- **RX** — usually idles high but flat (no transmit traffic from the device).

- Confirm the **baud rate** (115200 is common; also 9600/57600) and connect a

USB-TTL adapter (3.3 V!) TX→RX / RX→TX / GND→GND. A logic analyzer auto-bauds

from the bit timing.

### Q: How do I tell SPI flash apart from I²C EEPROM at a glance?

**A:** Both are commonly 8-pin, but:

- **25-series marking → SPI NOR** (6 signal pins: CS, CLK, DI, DO, plus WP/HOLD).

- **24-series marking → I²C EEPROM** (only two bus lines: **SCL, SDA**, plus

address pins A0–A2 and WP).

- Count the active bus lines and check the marking prefix. When unsure, the

datasheet pinout resolves it immediately.

---

## 4. Printed Circuit Board (PCB) Standards & Interpretation

### Q: What PCB/electronics standards should an assessor recognize?

**A:** You don't need to be a board designer, but recognizing these helps you

read a board and judge build quality:

- **IPC-2221** — generic PCB design standard (spacing, trace width, clearances).

- **IPC-A-600** — acceptability of *bare* boards (what a good/defective board

looks like).

- **IPC-A-610** — acceptability of *assembled* boards (solder joints, component

placement) — the most commonly cited workmanship standard.

- **IPC-7351** — land pattern (footprint) standards for SMD parts.

- **IPC-2581 / ODB++ / Gerber (RS-274X)** — PCB fabrication data formats; if you

ever get board design files, these are how they arrive.

- **J-STD-001** — soldering requirements (often paired with IPC-A-610).

- **IPC-4761** — via protection/tenting types (relevant when vias are your

probe points).

- **UL 796 / UL 94** — PCB safety and flammability ratings (the `94V-0` you see

silkscreened).

### Q: What can silkscreen and board markings tell me?

**A:** The silkscreen (usually white legend) is free documentation:

- **Reference designators** — the component-type prefix is a de-facto standard:

- `U` = IC, `Q` = transistor, `D` = diode, `R` = resistor, `C` = capacitor,

`L` = inductor, `Y`/`X` = crystal/oscillator, `J`/`P` = connector/header,

`TP` = **test point**, `SW` = switch, `F` = fuse, `FB` = ferrite bead.

- `TP` designators and named pads (`TX`, `RX`, `TCK`, `TMS`, `CLK`, `MOSI`)

are gifts — sometimes the debug interface is literally labeled.

- **Pin-1 indicators** — a square pad, a dot, a `1`, or a chamfer on the

silkscreen outline marks pin 1 of an IC or connector.

- **Board revision / part number / date code** — for tracking hardware

revisions and correlating with firmware.

- **Manufacturer marks, UL file number, and the `94V-0` flammability rating.**

- **Polarity marks** — `+` on electrolytics/tantalums, a band/stripe for diode

cathode.

### Q: How do layer count and construction affect my assessment?

**A:**

- **2-layer (top/bottom)** boards let you visually trace most signals — easy to

follow a debug pad to the CPU.

- **Multilayer (4/6/8+)** boards bury signals on inner layers; buses "disappear"

under a BGA and reappear elsewhere. You'll rely on the CPU datasheet and

probing rather than visual tracing.

- **BGA packages** hide all pins under the chip — no direct probing without

X-ray or removing the part. Debug/serial pads broken out to the surface become

your only practical access.

- **Blind/buried vias** (per IPC via classifications) mean a via on top may not

reach the layer you expect — don't assume a visible via is a through-connection.

- **Test points and unpopulated footprints** are the assessor's best friend:

manufacturing/JTAG test access is frequently left on the production board.

### Q: How do I safely probe a live board?

**A:**

- **Establish a common ground first** — always reference your probe/adapter GND

to the board GND before touching signals.

- **Know the logic level** (1.8 / 3.3 / 5 V) before connecting active tools;

measure VCC rails first. Mismatched levels damage parts.

- **Use the right tools** — multimeter (continuity, voltage), logic analyzer

(decode SPI/I²C/UART), oscilloscope (find active TX, check signal integrity),

SOIC clip (in-circuit flash), JTAGulator (pinout discovery).

- **Hold the CPU in reset** when reading a shared bus (SPI flash) so it doesn't

contend.

- **Don't back-power the board** through a signal pin; be deliberate about which

rails you drive.

---

## 5. Runtime, Protocol & Reporting

### Q: What should protocol/runtime testing focus on for OT?

**A:**

- **Engineering protocols** — many (legacy EtherNet/IP, Modbus, DNP3 without

SAv5, S7comm, PCCC) have **no authentication**: anyone routable can read/write

registers, change logic, or stop the CPU. Test whether write/program/stop

operations are authenticated at all.

- **Firmware/logic upload path** — can you push unsigned logic or firmware?

- **Web/management interfaces** — default creds, session handling, injection.

- **Fuzzing** — protocol parsers, but **only on a bench unit**; malformed

packets crash real PLCs.

- **Denial of resilience** — how does the device behave under load, malformed

input, or bus faults? (Availability is king in OT.)

### Q: How should findings be framed differently for OT?

**A:**

- **Lead with physical/process consequence**, not just CIA — "unauthenticated

write to coil 40001 opens the effluent valve" beats "improper access control."

- **Use environmental CVSS** to reflect the real deployment (safety, exposure,

compensating controls).

- **Map to ISA/IEC 62443-4-2 requirements and MITRE ATT&CK for ICS techniques**

so asset owners can act.

- **Give a compensating control** for every finding — patching is often

impossible, so network segmentation, unidirectional gateways, physical access

control, and monitoring are the realistic mitigations.

- **Coordinate disclosure** through CISA / the vendor PSIRT; OT vulns often get

ICS-CERT advisories and long remediation timelines.

---

## Quick Reference — Interface Pin Cheat Sheet

| Interface | Signals to find | Typical footprint | Discovery tool |

|-----------|-----------------|-------------------|----------------|

| **UART** | TX, RX, GND, (VCC) | 4 pads/header | Scope/logic analyzer (find active TX at boot) |

| **JTAG** | TCK, TMS, TDI, TDO, (TRST), GND | 10/14/20-pin or via grid | JTAGulator / JTAGenum |

| **SWD** | SWCLK, SWDIO, GND | 2 pads + gnd near ARM CPU | JTAGulator, CPU datasheet |

| **SPI flash** | CS#, CLK, DI, DO, GND, VCC | 8-pin SOIC/WSON, "25" prefix | SOIC clip + flashrom |

| **I²C EEPROM** | SCL, SDA, GND, VCC | 8-pin SOIC, "24" prefix | Bus Pirate / logic analyzer |

## Common Tools

- **Firmware:** `binwalk`, `unblob`, `firmwalker`, Ghidra/IDA, `flashrom`,

a composition-analysis/SBOM tool, `strings`/`grep`.

- **Hardware:** multimeter, logic analyzer (Saleae/clone), oscilloscope,

USB-TTL 3.3 V adapter, SOIC-8 test clip, CH341A/Bus Pirate/FT2232 programmer,

JTAGulator, soldering/rework station, magnification.

- **Protocol:** Wireshark (with ICS dissectors), vendor engineering software on

an isolated bench, protocol-specific fuzzers.

---

*Scope reminder: bench-test owned or authorized devices only. Coordinate any

real vulnerability through CISA/vendor PSIRT disclosure processes. Nothing here

authorizes testing against production OT or third-party equipment.*